| DIVISIONE RIELLO TRADE Via degli Alpini, 1 37045 Legnago VR |  |
| --- | --- |

VERTICAL DEFLECTORS KIT

These instructions are an integral part of the booklet of the appliance on which the accessory has been installed. Please refer to the GENERAL SAFETY INFORMATION and PRECAUTIONS in this booklet.

KIT DESCRIPTION

| The vertical deflectors kit allows you to change the air output angle in a horizontal direction in order to improve the distribution. It is identified with various codes depending on the model of the air heater used with it. | KIT CODE; ACU. 4155626; 12M-13M. 4155628; 22M-23M. 4155630; 32M-33M. 4155632; 42M-43M. 4155634; 52M-53M. 4155636; 62M-63M. 4155638; 72T -73T. 4155640; 82T -83T. 4155642; 92T -93T. |
| --- | --- |

PACKAGE CONTENTS

| # Description Q.ty Vertical deflector grilles 1 (mod. 12M – 73T) 2 (mod. 82T – 93T) Self-tapping screw 3 (only mod. 82T – 93T) Instructions 1 | mod. 12M-73T | mod. 82T-93T |
| --- | --- | --- |

INSTALLATION

| The assembly should be carried out by qualified personnel - Assemble the vertical deflector grilles (only mod. 82T-93T) using the screws supplied |  |
| --- | --- |

| - Manually open the vertical deflectors to get the desired angle of opening | 20° min 20° min |
| --- | --- |

| - Remove the horizontal deflector grille |  |
| --- | --- |

| - Remove the 4 self-tapping screws indicated in the figure |  |
| --- | --- |

| - Put the vertical deflector grille in place and fix it with the 4 screws that were removed |  |
| --- | --- |

| - Put back the horizontal deflector grille proceeding in the reverse order |  |
| --- | --- |

| Use adequate personal protection equipment (gloves, etc.) |  |
| --- | --- |

CONTROL

When the installation is finished:

-	Check the air is distributed correctly and if necessary correct it by changing the angle of opening of the vertical and/or horizontal deflectors.